Steps for Packaging and Shipping Frozen Specimens using Dry Ice

- Assemble all materials for packing and shipping frozen samples, including specimen box, specimen shipping list, Ziplock bags, Styrofoam shipping box, dry ice and other packing materials.   Make sure the labels on the specimen shipping list match the labeled cryovials in the specimen box.

- Place a rubber band around the pre-labeled grid box, and then wrap it with absorbent paper.   Place the specimen box into a Ziplock bag and seal the bag.

- Place the Ziplock bag into the bottom of the shipping box.  If necessary, use sheets of bubble wrap to ensure specimens remain in an upright position.

- Fill the Styrofoam-lined shipping box with dry ice.  Allow one pound of dry ice for every 2 hours in transport.  To protect your hands from burning and to protect your eyes from ice shards, use heavy gloves and safety glasses when filling the shipping box with dry ice.  The following link is useful for locating dry ice suppliers throughout the world: http://www.dryicedirectory.com/world.htm

- Fill any empty space in the box with bubble wrap or paper.  This will help prevent shifting of the specimen box when the ice dissipates.

- Place the Styrofoam lid on top of the shipping box.  Place the completed specimen shipment list in a Ziplock bag and attach to the top of the Styrofoam lid.  Remember to photocopy the specimen shipment list and retain a copy for your records.

- Secure the outer lid of the shipping box with tape.  When using dry ice, the packaging must permit the release of carbon dioxide gas to prevent a build-up of pressure that could rupture the package.  Leaving an air gap when taping will ensure that carbon dioxide is released.

- Label the shipping box with a dry ice sticker.

- Use a pre-printed label or marker to label each shipping box with the address of the recipient lab.  Include contact name, telephone number and return address information.  Contact the recipient lab before shipping the specimens to ensure that the staff is ready to receive the shipment.

Use of trade names and commercial sources is for identification only and does not imply an endorsement by the U.S. Department of Health and Human Services (DHHS) and the Centers for Disease Control and Prevention (CDC).  References to non-CDC sites on the Internet are provided as a service and do not constitute or imply endorsement of these organizations or their programs or products by the DHHS and CDC.  CDC is not responsible for the content of these sites nor can DHHS and CDC warranty the reliability and quality of the products listed on this site.